Lab 1 Summary

Schuster, Mark

Section 1540

09/12/2017

Pre-Lab Answers:

None

Problems encountered:

None in particular. All documents and resources were clear and informative!

Future Work/Applications:

I really enjoyed using the LED backpack and I look forward to using the others.

Schematics:

Not applicable for this lab.

Pseudocode/Flowcharts:

Switch circuit diagram:

Part A flow diagram:

Part B pseudocode:

Part B waveform:

Part C pseudocode:

Program Code:

Part A code:

; Lab 1 part A

; Name:         Mark L. Schuster

; Section #:    1540

; TA Name:      Christopher Crary

; Description:  This assembly application reads the current state of

;               the switches on the 'SWITCH & LED BACKPACK' and turns

;               on the corresponding LED.

.nolist                           ; Included for fun.

.include "ATxmega128A1Udef.inc"   ;

.list                             ;

.def initVal = r16                ; Will hold the port DIRSET vlaues.

.def switchValues = r17           ; Will hold the switch's input value.

.equ ALL_IN = 0x00                ; 8-bit vector that will set an 8-bit GPIO port to input.

.equ ALL_OUT = 0xFF               ; 8-bit vector that will set an 8-bit GPIO port to output.

.org 0x0000

rjmp init                     ; Start at 0x0000 and jump to program.

.org 0x200                        ; Start functionality at 0x200 to avoid precious interupt vectors.

init:

ldi initVal, ALL_IN           ; Set 'initValue' to 8-bit GPIO input vector.

sts PORTA_DIRSET, initVal     ; Set PORTA to be input.

ldi initVal, ALL_OUT          ; Set 'initValue' to 8-bit GPIO output vector.

sts PORTC_DIRSET, initVal     ; Set PORTC to be output.

rjmp main

main:

lds switchValues, PORTA_IN    ; Load the switch's value into the 'switchValues'.

sts PORTC_OUT, switchValues   ; Load 'switchValues' into the output to the LEDs.

rjmp main                     ; Loop for continued updating.

Part B code:

; Lab 1 part B

; Name:         Mark L. Schuster

; Section #:    1540

; TA Name:      Christopher Crary

; Description:  This assembly application causes a LED to blink

;				with variable period using delay subroutines.

.nolist                           ; Included for fun.

.include "ATxmega128A1Udef.inc"   ;

.list                             ;

.def initVal = r16                ; Will hold the port DIRSET vlaues.

.def ledToggle = r17              ; Will hold the value for the port output toggle.

.def countA = r18                 ; Will hold the count for the outer busy loop.

.def countB = r19                 ; Will hold the count for the inner busy loop.

.def delayTime = r20              ; Will hold the scaler for the number of times the 10ms delay is to occur.

.def temp = r21                   ; Will hold the count for how many 10ms delays have occurred.

.equ ALL_OUT = 0xFF               ; 8-bit vector that will set an 8-bit GPIO port to output.

.equ LED_OFF = 0xFF               ; Used to initialize the LEDs to off.

.equ LED_TOGGLE = 0x01            ; Used to toggle the right-most LED.

.equ numberOfDelays = 127

.org 0x0000

rjmp init                     ; Start at 0x0000 and jump to program.

.org 0x200

init:

ldi initVal, ALL_OUT          ; Store the 8-bit output vector in 'initVal'.

sts PORTC_DIRSET, initVal     ; Set portc to an output.

ldi initVal, LED_OFF          ; Store the initial LED state in 'initVal'.

sts PORTC_OUT, initVal        ; Set the LEDs to the initial value.

ldi ledToggle, LED_TOGGLE     ; Set the LED toggle value.

ldi delayTime, numberOfDelays ; Set the arbitrary number of 10ms delays.

rjmp main                     ;

main:

sts PORTC_OUTTGL, ledToggle   ; Toggle the LED.

rcall DELAYx10ms              ; Call DELAYx10ms.

rjmp main                     ; Repeat.

.org 0x250

;*********************SUBROUTINES**************************************

; Subroutine Name: DELAYx10ms

; Causes the device to pause operation for delayTime x 10ms.

; Inputs: delayTime

; Ouputs: None

; Affected: None

DELAYx10ms:

push delayTime                ; Store the contents of delayTime.

push temp                     ; Store the contents of temp.

ldi temp, 0x00                ; Initialize temp to zero.

delayXLoop:

cp temp, delayTime            ; Compare current count of delays and the total number of delays.

breq exitDELAYx10ms           ; Exit if they are equal.

rcall DELAY_10ms              ; If not, perform a 10ms delay.

inc temp                      ; Increment the count.

rjmp delayXLoop               ; Repeat.

exitDELAYx10ms:

pop temp                      ; Restore temp.

pop delayTime                 ; Restore delayTime.

ret                           ; Return.

.org 0x300

;*********************SUBROUTINES**************************************

; Subroutine Name: DELAY_10ms

; Causes the device to pause operation for 10ms.

; Inputs: None

; Ouputs: None

; Affected: None

DELAY_10ms:

push countA                   ; Store the contents of countA.

push countB                   ; Store the contents of countB.

ldi countA, 0x00              ; Initialize countA to zero.

ldi countB, 0x00              ; Initialize countB to zero.

outerLoop:

cpi countA, 0x08              ; Compare countA with 0x08.

breq exitDELAY_10ms           ; If equal, exit.

innnerLoop:

cpi countB, 0xFD              ; If not, compare countB with 0xFD.

breq incCountA                ; If equal, exit the inner loop.

inc countB                    ; Increment countB.

rjmp innnerLoop               ; Return to the nested loop.

incCountA:

ldi countB, 0x00              ; Reset countB.

inc countA                    ; Increment countA.

rjmp outerLoop                ; return to the outer loop.

exitDELAY_10ms:

pop countB                    ; Restore countB.

pop countA                    ; Restore countA.

ret                           ; Return.

Part C code:

; Lab 1 part C

; Name:         Mark L. Schuster

; Section #:    1540

; TA Name:      Christopher Crary

; Description:  This assembly application is a game

;               which is played by trying to press S2

;               when LEDs 3 and 4 are lit.

.nolist                           ; Included for fun.

.include "ATxmega128A1Udef.inc"   ;

.list                             ;

.def initVal = r16                ; Will hold the port DIRSET values.

.def valLED = r17                 ; Will hold the LED value.

.def countA = r18                 ; Will hold the count for the outer busy loop.

.def countB = r19                 ; Will hold the count for the inner busy loop.

.def delayTime = r20              ; Will hold the scaler for the number of times the 10ms delay is to occur.

.def temp = r21                   ; Will hold the count for how many 10ms delays have occurred.

.equ BLACK = 0xFF                 ; Denotes the value of black for the RGB LED.

.equ RED = 0xEF                   ; Denotes the value of red for the RGB LED.

.equ GREEN = 0xDF                 ; Denotes the value of green for the RGB LED.

.equ ALL_IN = 0x00                ; 8-bit vector that will set an 8-bit GPIO port to input.

.equ ALL_OUT = 0xFF               ; 8-bit vector that will set an 8-bit GPIO port to output.

.equ LED_INIT = 0x3F              ; Denotes the starting configuration of PortC's LEDs.

.equ numberOfDelays = 8           ; Use to create the 80ms delay specified in the prompt.

.org 0x0000

rjmp init                     ; Start at 0x0000 and jump to program.

.org 0x200

init:

ldi initVal, ALL_OUT          ; Set 'initValue' to 8-bit GPIO output vector.

sts PORTC_DIRSET, initVal     ; Set PORTC to be output.

sts PORTD_DIRSET, initVal     ; Set PORTD to be output.

ldi initVal, LED_INIT         ; Store the initial LED state in 'initVal'.

sts PORTC_OUT, initVal        ; Set the LEDs to the initial value.

ldi initVal, ALL_IN           ; Set 'initValue' to 8-bit GPIO input vector.

sts PORTF_DIRSET, initVal     ; Set PORTF to be output.

ldi valLED, LED_INIT          ; Load the inital LED configuration into 'valLED'.

ldi delayTime, numberOfDelays ; Set the arbitrary number of 10ms delays.

rjmp main

main:

rcall DELAYx10ms              ; Call the 80ms delay.

mov temp, valLED              ; Move the LED value into 'temp' to be processed. The program

andi temp, 0x01               ; will check if the least significant bit is set to determine if

cpi temp, 0x00                ; the carry flag should be set or cleared for the impending rotation.

breq noCarry                  ; If the least significant bit is clear, then the carry flag will be cleared.

sec                           ; If not, set the carry flag.

rjmp loadLED                  ; Jump to pulling the LEDs current value.

noCarry:

clc                           ; Clear the carry flag.

loadLED:

ror valLED                    ; Perform a bit-wise rotation right on the LEDs' value.

sts PORTC_OUT, valLED         ; Push the rotated value to the output.

checkForS2:

lds temp, PORTF_IN            ; Pull the button values into 'temp'.

andi temp, 0x08               ; And 'temp' and 0x08 to check if S2 is being pressed.

cpi temp, 0x08                ; Compare 'temp' and 0x08.

breq loop                     ; If they're equal S2 is not being pressed, thus proceed.

checkIfWin:

mov temp, valLED              ; Now the program will check if a win has occurred.

andi temp, 0xE7               ; This is done by checking if the LEDs are in the correct position.

cpi temp, 0xE7                ; Compare 'temp' and 0xE7 to check if it is the correct value.

breq win                      ; If it is, move to 'win'.

loss:

ldi temp, RED                 ; If not, set the RGB LED to red.

sts PORTD_OUT, temp           ; Push RED to the RGB LED.

rjmp startOver                ; Jump to the end of game.

win:

ldi temp, GREEN               ; Set the RGB LED to green.

sts PORTD_OUT, temp           ; Push GREEN to the RGB LED.

rjmp startOver                ; Jump to the end of game.

loop:

ldi temp, BLACK               ; If S2 is not pressed or the program returned from the

sts PORTD_OUT, temp           ; end of game, reset the RGB LED to off.

rjmp main                     ; Jump to the beginning.

startOver:

lds temp, PORTF_IN            ; Once S2 was pressed, the program waits till S1 is pressed

andi temp, 0x04               ; to reset the game.

cpi temp, 0x04                ; After pulling the button value's, compare them.

brne loop                     ; If S1 has been pressed, jump back to main loop.

rjmp startOver                ; If not, keep checking.

.org 0x300

;*********************SUBROUTINES**************************************

; Subroutine Name: DELAYx10ms

; Causes the device to pause operation for delayTime x 10ms.

; Inputs: delayTime

; Ouputs: None

; Affected: temp

DELAYx10ms:

push delayTime                ; Store the contents of delayTime.

push temp                     ; Store the contents of temp.

ldi temp, 0x00                ; Initialize temp to zero.

delayXLoop:

cp temp, delayTime            ; Compare current count of delays and the total number of delays.

breq exitDELAYx10ms           ; Exit if they are equal.

rcall DELAY_10ms              ; If not, perform a 10ms delay.

inc temp                      ; Increment the count.

rjmp delayXLoop               ; Repeat.

exitDELAYx10ms:

pop temp                      ; Restore temp.

pop delayTime                 ; Restore delayTime.

ret                           ; Return.

.org 0x350

;*********************SUBROUTINES**************************************

; Subroutine Name: DELAY_10ms

; Causes the device to pause operation for 10ms.

; Inputs: None

; Ouputs: None

; Affected: None

DELAY_10ms:

push countA                   ; Store the contents of countA.

push countB                   ; Store the contents of countB.

ldi countA, 0x00              ; Initialize countA to zero.

ldi countB, 0x00              ; Initialize countB to zero.

outerLoop:

cpi countA, 0x08              ; Compare countA with 0x08.

breq exitDELAY_10ms           ; If equal, exit.

innnerLoop:

cpi countB, 0xFD              ; If not, compare countB with 0xFD.

breq incCountA                ; If equal, exit the inner loop.

inc countB                    ; Increment countB.

rjmp innnerLoop               ; Return to the nested loop.

incCountA:

ldi countB, 0x00              ; Reset countB.

inc countA                    ; Increment countA.

rjmp outerLoop                ; return to the outer loop.

exitDELAY_10ms:

pop countB                    ; Restore countB.

pop countA                    ; Restore countA.

ret                           ; Return.

Appendix:

- lab1.pdf

- lab1a.asm

- lab1b.asm

- lab1c.asm