Lab 2 Summary

Schuster, Mark

Section 1540

09/30/2017

B) Pre-Lab Answers:

- 2 MHz

- To set the CLK to 16 MHz:

  - Enable the 32Mhz CLK

  - Wait until its enable flag is set

  - Write the “IOREG” signature to CPU_CCP

  - Set the prescaler to 2 to divide 32MHz by 2

  - Write the “IOREG” signature to CPU_CCP again

  - Select the 32Mhz CLK

- One would use the phase locked loop (PLL) to augment the 32Mhz by a factor of 4 to produce a 128Mhz CLK. Then this would be selected as the input for the system CLK. Prescaler A would be set to 1 and the resulting CLKPER4 would be at a frequency of 128MHz. Then prescaler B would be set to 2 which would result in CLKPER2 outputting a CLK signal at 64Mhz. Finally, prescaler C would be set to 2 which would result in the CPU’s clock running at 32Mhz.

- Timers are useful because they let the CPU work on another process without having to constantly decrement a counter. Thus, when a timer is up, an interrupt can be triggered and the CPU can then immediately operate on a separate task.

- From 4k to 16k or 0x004000 to 0xFFFFFF

- It is called 3 port ALE1 because is uses the pins of three ports (Ports J, K, And H) to output the proper addresses. It is AEL1 because a latch is used for multiplexing between bytes 0 and 1 of the SRAM address.

- As stated in my answer for problem 6, ports J, K, and H are used for the 3-port setup. Port K is used to transmit bits [7:0] and [15:8] of the SRAM address as determined by whether the AEL signal is true or not. Port J only transmits bits [7:0] of the SRAM address. Finally, port H is used to indicate what actions are being taken by the EBI along with bits [19:16] of the SRAM address. Bit 0 represents the Write signal, Bit 1 represents the Read signal, and Bit 2 represents the ALE signal. Bit 3 is unused, and bits [7:4] are multiplexed between the chip select signals and bits [19:16] of the SRAM address.

- 256 Kilobits. I looked up the data sheet, and then calculated for safe measure: 15 address pins*1 byte per address→215=32,768 bytes per address or a 32KB SRAM

- No for 32K as this is the size of our SRAM and all address lines will be used. Yes for 64K as there will be one extra bit, meaning 21 or 2 aliases.

C) Problems encountered:

At first, I was unable to select the 32MHz clock, but I realized that the CPU_CPP reg has to have “IOREG” written to it directly before both the prescaler and clock control are written to.

D) Future Work/Applications:

I really enjoyed using the interrupts and I look forward to the next lab where we focus on them to a greater extent.

E) Schematics:

Not applicable for this lab.

F) Pseudocode/Flowcharts:

Part A Pseudocode:

Part B Pseudocode:

Part C Pseudocode:

Part D Pseudocode:

G) Program Code:

lab2a.asm:

; Lab 2 part A

; Name:         Mark L. Schuster

; Section #:    1540

; TA Name:      Christopher Crary

; Description:  Sets the system clock to the 32Mhz clock

;               with a given prescaler.

.nolist                           ; Included for fun.

.include "ATxmega128A1Udef.inc"   ;

.list                             ;

.def clkPrescaler = r17           ; Denotes the input for the USE32MHzCLK input. Will hold the prescaler value.

.equ CLKEN = 0b0010               ; Enables the 32Mhz CLK.

.equ IOREG = 0xD8                 ; The value that sets the CPU_CCP reg to 'IOREG' mode.

.equ CLKPS = 0b00001100           ; Value that sets Prescaler A to 4.

.equ CLKSEL = 1                   ; Value to select the 32MHz CLK.

.equ CLKOUT = 0b00000001          ; Value to output the CLK signal to port C.

.equ MSBOUT = 0b10000000          ; Value to set the MSB of a port to output.

.org 0x0000

rjmp init                     ; Start at 0x0000 and jump to program.

.org 0x200

init:

ldi clkPrescaler, CLKPS       ; Load the prescaler value and call USE32MHzCLK.

rcall USE32MHzCLK             ;

main:

ldi r16, MSBOUT               ; Set the MSB of port C to output.

sts PORTC_DIRSET, r16         ;

ldi r16, CLKOUT               ; Output the CLK signal to port C,

sts PORTCFG_CLKEVOUT, r16     ;

run:

jmp run                       ; Loop endlessly.

.org 0x300

;*********************SUBROUTINES**************************************

; Subroutine Name: USE32MHzCLK

; Sets the external 32MHz as the active clock for the device

; Inputs: r17 as the desired prescaler for the clock

; Outputs: None

; Affected: r16, r17

USE32MHzCLK:

push r16                      ; Preserve the values of r16, r17.

push r17                      ;

ldi r16, CLKEN                ; Load the CLK enable value and store it in the CLK control.

sts OSC_CTRL, r16             ;

checkReady:

lds r16, OSC_STATUS           ; This section pulls the oscillator status reg and constantly

andi r16, CLKEN               ; checks if the 32Mhz CLK is ready yet.

cpi r16, CLKEN                ;

breq clockSel                 ; If it is move on, if not loop continuously.

rjmp checkReady               ;

clockSel:

ldi r16, IOREG                ; Write 'IOREG' to the CPU_CCP to allow the CLK Prescaler

sts CPU_CCP, r16              ; to be written to.

sts CLK_PSCTRL, clkPrescaler  ;

sts CPU_CCP, r16              ; Write 'IOREG' to the CPU_CCP to allow the CLK Control

ldi r16, CLKSEL               ; to be set to output the 32 MHz.

sts CLK_CTRL, r16             ;

pop r17                       ; Restore the values of r16 and r17.

pop r16                       ;

ret                           ; return.

lab2b.asm:

; Lab 2 part B

; Name:         Mark L. Schuster

; Section #:    1540

; TA Name:      Christopher Crary

; Description:  Setup a timer/counter to run at 1024 times the period

;				of the system clock and out put the lower byte of the

;				count to be analyzed.

.nolist                           ; Included for fun.

.include "ATxmega128A1Udef.inc"   ;

.list                             ;

.def clkPrescaler = r17           ; Denotes the input for the USE32MHzCLK input. Will hold the prescaler value.

.equ CLKEN = 0b0010               ; Enables the 32Mhz CLK.

.equ IOREG = 0xD8                 ; The value that sets the CPU_CCP reg to 'IOREG' mode.

.equ CLKPS = 0b00001100           ; Value that sets Prescaler A to 4.

.equ CLKSEL = 1                   ; Value to select the 32MHz CLK.

.equ CLKOUT = 0b00000001          ; Value to output the CLK signal to port C.

.equ TCSEL = 0b0111               ; Value to set the prescaler of the TC to be 1024 time the period of the system CLK.

.equ TCPER = 0x00FF               ; Value of the TC period.

.equ ALLOUT = 0xFF                ; 8-bit vector that will set an 8-bit GPIO port to output.

.org 0x0000

rjmp init                     ; Start at 0x0000 and jump to program.

.org 0x200

init:

ldi clkPrescaler, CLKPS       ; Load the prescaler value and call USE32MHzCLK.

rcall USE32MHzCLK             ;

counterInit:

ldi r16, TCSEL                ; Enable the TC and set its period to be 1024 times that of the system CLK.

sts TCC2_CTRLA, r16           ;

ldi r16, low(TCPER)           ; Load the period of the TC into the TC's period regs.

sts TCC2_LPER, r16            ;

ldi r16, high(TCPER)          ;

sts TCC2_HPER, r16            ;

portcInit:

ldi r16, ALLOUT               ; Set port C to output.

sts PORTC_DIRSET, r16         ;

main:

lds r16, TCC2_LCNT            ; Load the lower byte of the TC's count and output it

sts PORTC_OUT, r16            ; to port C.

rjmp main                     ; Repeat that action forever.

.org 0x300

;*********************SUBROUTINES**************************************

; Subroutine Name: USE32MHzCLK

; Sets the external 32MHz as the active clock for the device

; Inputs: r17 as the desired prescaler for the clock

; Outputs: None

; Affected: r16, r17

USE32MHzCLK:

push r16                      ; Preserve the values of r16, r17.

push r17                      ;

ldi r16, CLKEN                ; Load the CLK enable value and store it in the CLK control.

sts OSC_CTRL, r16             ;

checkReady:

lds r16, OSC_STATUS           ; This section pulls the oscillator status reg and constantly

andi r16, CLKEN               ; checks if the 32Mhz CLK is ready yet.

cpi r16, CLKEN                ;

breq clockSel                 ; If it is move on, if not loop continuously.

rjmp checkReady               ;

clockSel:

ldi r16, IOREG                ; Write 'IOREG' to the CPU_CCP to allow the CLK Prescaler

sts CPU_CCP, r16              ; to be written to.

sts CLK_PSCTRL, clkPrescaler  ;

sts CPU_CCP, r16              ; Write 'IOREG' to the CPU_CCP to allow the CLK Control

ldi r16, CLKSEL               ; to be set to output the 32 MHz.

sts CLK_CTRL, r16             ;

pop r17                       ; Restore the values of r16 and r17.

pop r16                       ;

ret                           ; return.

lab2c.asm:

; Lab 2 part C

; Name:         Mark L. Schuster

; Section #:    1540

; TA Name:      Christopher Crary

; Description:  Initialize the SRAM's mapped address to 0x200000 using the EBI

.nolist                           ; Included for fun.

.include "ATxmega128A1Udef.inc"   ;

.list                             ;

.equ EBIBASEADDR = 0x200000       ; Value of the base address of where the external SRAM is to be mapped.

.equ EBIINIT = 0b01000001         ; Value to set the EBI to the 3-port, AEL1 with chip select mode and 8 bit reg sizes.

.equ ALL_OUT = 0xFF               ; 8-bit vector that will set an 8-bit GPIO port to output.

.equ ALL_IN = 0x00                ; 8-bit vector that will set an 8-bit GPIO port to input.

.equ EBI_PORTH_OUT = 0b11110111   ; 8-bit vector that will set all bits of port H as output aside from bit 3.

.equ EBISRAMEN = 0b00011101       ; Value to set the EBI to SRAM mode with a 32K address space.

.org 0x0000

rjmp init                     ; Start at 0x0000 and jump to program.

.org 0x200

init:

ldi ZL, byte3(EBIBASEADDR)    ; Load the SRAM base address into both the Z and Y pointers.

out CPU_RAMPZ, ZL             ;

ldi ZH, byte2(EBIBASEADDR)    ;

ldi ZL, byte1(EBIBASEADDR)    ;

ldi YL, byte3(EBIBASEADDR)    ;

out CPU_RAMPY, YL             ;

ldi YH, byte2(EBIBASEADDR)    ;

ldi YL, byte1(EBIBASEADDR)    ;

rcall INITEBI                 ; Call the INITEBI subroutine.

loop:

ldi r16, 0xA5                 ; Load 0xa5 into a reg and write it to the SRAM.

st Y, r16                     ;

ld r17, Z                     ; Load the written value back from the SRAM.

rjmp loop                     ; Loop endlessly.

.org 0x300

;*********************SUBROUTINES**************************************

; Subroutine Name: INITEBI

; Initialized the EBI

; Inputs: None

; Outputs: None

; Affected: r16, Z

INITEBI:

push r16                       ; Preserve r16 and the Z pointer.

mov r16, ZL                    ;

push r16                       ;

mov r16, ZH                    ;

push r16                       ;

lds r16, CPU_RAMPZ             ;

push r16                       ;

ldi r16, ALL_IN                ; Set port J to input.

sts PORTJ_DIRSET, r16          ;

ldi r16, ALL_OUT               ; Set port K to output.

sts PORTK_DIRSET, r16          ;

ldi r16, EBI_PORTH_OUT         ; Set all port H to output expect for bit 3 which goes unused.

sts PORTH_DIRSET, r16          ;

ldi r16, EBIINIT               ; Set the EBI to the 3-port, AEL1 with chip select mode with 8 bit reg sizes.

sts EBI_CTRL, r16              ;

ldi ZL, 0x00                   ; Clear Z's ramp, and set Z to the address of chip 0's base address.

out CPU_RAMPZ, ZL              ;

ldi ZL, low(EBI_CS0_BASEADDR)  ;

ldi ZH, high(EBI_CS0_BASEADDR) ;

ldi r16, byte2(EBIBASEADDR)    ; Then load the chosen base address into chip 0's base address's location.

st Z+, r16                     ;

ldi r16, byte3(EBIBASEADDR)    ;

st Z, r16                      ;

ldi r16, EBISRAMEN             ; Specify that chip 0 is SRAM with a 32K address space.

sts EBI_CS0_CTRLA, r16         ;

pop r16                        ; Restore r16 and the Z pointer.

mov ZL, r16                    ;

out CPU_RAMPZ, r16             ;

pop r16                        ;

mov ZH, r16                    ;

pop r16                        ;

mov ZL, r16                    ;

pop r16                        ;

ret                            ; Return.

lab2d.asm:

; Lab 2 part D

; Name:         Mark L. Schuster

; Section #:    1540

; TA Name:      Christopher Crary

; Description:  Program that establishes external SRAM at 0x200000 and writes the current switch values to the SRAM

;               every second. Additionally, the current value at the location of the stored switch values is outputted

;               to the LEDs every second.

.nolist                           ; Included for fun.

.include "ATxmega128A1Udef.inc"   ;

.list                             ;

.equ EBIBASEADDR = 0x200000       ; Value of the base address of where the external SRAM is to be mapped.

.equ EBIINIT = 0b01000001         ; Value to set the EBI to the 3-port, AEL1 with chip select mode and 8 bit reg sizes.

.equ ALL_OUT = 0xFF               ; 8-bit vector that will set an 8-bit GPIO port to output.

.equ ALL_IN = 0x00                ; 8-bit vector that will set an 8-bit GPIO port to input.

.equ EBI_PORTH_OUT = 0b11110111   ; 8-bit vector that will set all bits of port H as output aside from bit 3.

.equ EBISRAMEN = 0b00011101       ; Value to set the EBI to SRAM mode with a 32K address space.

.def clkPrescaler = r17           ; Denotes the input for the USE32MHzCLK input. Will hold the prescaler value.

.equ CLKEN = 0b0010               ; Enables the 32Mhz CLK.

.equ IOREG = 0xD8                 ; The value that sets the CPU_CCP reg to 'IOREG' mode.

.equ CLKPS = 0b00000000           ; Value that sets Prescaler A to 4.

.equ CLKSEL = 1                   ; Value to select the 32MHz CLK.

.equ CLKOUT = 0b00000001          ; Value to output the CLK signal to port C.

.equ TCSEL = 0b0111               ; Value to set the prescaler of the TC to be 1024 time the period of the system CLK.

.equ TCPER = 0x7D00               ; Value of the TC period.

.equ TCCMPAINT = 0b00010000       ; Value to init the TC compare A reg.

.equ TCCMPA_INTFLAGLOC = 4        ; Location of the compare interrupt flag in the TC's interrupt flags reg.

.org 0x0000

rjmp init                     ; Start at 0x0000 and jump to program.

.org 0x100

init:

sei                           ; Set the global interrupt bit.

rcall INITEBI                 ; Call INITEBI.

ldi clkPrescaler, CLKPS       ; Load the prescaler input for USE32MHzCLK and then call it.

rcall USE32MHzCLK             ;

rcall SETTIMER                ; Call SETTIMER.

portcInit:

ldi r16, ALL_IN               ; Set 'initValue' to 8-bit GPIO input vector.

sts PORTA_DIRSET, r16         ; Set PORTA to be input.

ldi r16, ALL_OUT              ; Set 'initValue' to 8-bit GPIO output vector.

sts PORTC_DIRSET, r16         ; Set PORTC to be output.

ldi ZL, byte3(EBIBASEADDR)    ; Load the base address of the external SRAM into the Z

out CPU_RAMPZ, ZL             ; and Y pointers.

ldi ZH, byte2(EBIBASEADDR)    ;

ldi ZL, byte1(EBIBASEADDR)    ;

ldi YL, byte3(EBIBASEADDR)    ;

out CPU_RAMPY, YL             ;

ldi YH, byte2(EBIBASEADDR)    ;

ldi YL, byte1(EBIBASEADDR)    ;

main:

lds r16, TCC2_INTFLAGS        ; Check the compare A interrupt flag of the TC.

mov r17, r16                  ;

andi r16, TCCMPAINT           ;

cpi r16, TCCMPAINT            ;

breq toggle                   ; If it is set, move to toggle.

rjmp main                     ; If not, loop to check again.

toggle:

cbr r17, TCCMPA_INTFLAGLOC    ; Clear the interrupt flag from the TC's interrupt flags reg.

sts TCC2_INTFLAGS, r17        ;

lds r16, PORTA_IN             ; Pull the current switch values and store them in the

st Y, r16                     ; external SRAM.

ld r17, Z                     ; Load the switch values back from SRAM and output them to

sts PORTC_OUT, r17            ; the LEDs.

rjmp main                     ; Loop to wait for the flag to be set again.

.org 0x300

;*********************SUBROUTINES**************************************

; Subroutine Name: USE32MHzCLK

; Sets the external 32MHz as the active clock for the device

; Inputs: r17 as the desired prescaler for the clock

; Outputs: None

; Affected: r16, r17

USE32MHzCLK:

push r16                      ; Preserve the values of r16, r17.

push r17                      ;

ldi r16, CLKEN                ; Load the CLK enable value and store it in the CLK control.

sts OSC_CTRL, r16             ;

checkReady:

lds r16, OSC_STATUS           ; This section pulls the oscillator status reg and constantly

andi r16, CLKEN               ; checks if the 32Mhz CLK is ready yet.

cpi r16, CLKEN                ;

breq clockSel                 ; If it is move on, if not loop continuously.

rjmp checkReady               ;

clockSel:

ldi r16, IOREG                ; Write 'IOREG' to the CPU_CCP to allow the CLK Prescaler

sts CPU_CCP, r16              ; to be written to.

sts CLK_PSCTRL, clkPrescaler  ;

sts CPU_CCP, r16              ; Write 'IOREG' to the CPU_CCP to allow the CLK Control

ldi r16, CLKSEL               ; to be set to output the 32 MHz.

sts CLK_CTRL, r16             ;

pop r17                       ; Restore the values of r16 and r17.

pop r16                       ;

ret                           ; return.

.org 0x350

;*********************SUBROUTINES**************************************

; Subroutine Name: INITEBI

; Initialized the EBI

; Inputs: None

; Outputs: None

; Affected: r16, Z

INITEBI:

push r16                      ; Preserve r16 and the Z pointer.

mov r16, ZL                   ;

push r16                      ;

mov r16, ZH                   ;

push r16                      ;

lds r16, CPU_RAMPZ            ;

push r16                      ;

ldi r16, ALL_IN               ; Set port J to input.

sts PORTJ_DIRSET, r16         ;

ldi r16, ALL_OUT              ; Set port K to output.

sts PORTK_DIRSET, r16         ;

ldi r16, EBI_PORTH_OUT        ; Set all port H to output expect for bit 3 which goes unused.

sts PORTH_DIRSET, r16         ;

ldi r16, EBIINIT              ; Set the EBI to the 3-port, AEL1 with chip select mode with 8 bit reg sizes.

sts EBI_CTRL, r16             ;

ldi ZL, 0x00                  ; Clear Z's ramp, and set Z to the address of chip 0's base address.

out CPU_RAMPZ, ZL             ;

ldi ZL, low(EBI_CS0_BASEADDR) ;

ldi ZH, high(EBI_CS0_BASEADDR);

ldi r16, byte2(EBIBASEADDR)   ; Then load the chosen base address into chip 0's base address's location.

st Z+, r16                    ;

ldi r16, byte3(EBIBASEADDR)   ;

st Z, r16                     ;

ldi r16, EBISRAMEN            ; Specify that chip 0 is SRAM with a 32K address space.

sts EBI_CS0_CTRLA, r16        ;

pop r16                       ; Restore r16 and the Z pointer.

mov ZL, r16                   ;

out CPU_RAMPZ, r16            ;

pop r16                       ;

mov ZH, r16                   ;

pop r16                       ;

mov ZL, r16                   ;

pop r16                       ;

ret                           ; Return.

.org 0x450

;*********************SUBROUTINES**************************************

; Subroutine Name: SETTIMER

; Initialized TCC2 by setting its period and compare A value.

; Inputs: None

; Outputs: None

; Affected: r16

SETTIMER:

push r16                      ; Preserve r16.

ldi r16, TCSEL                ; Enable the TC and set its period to be 1024 times that of the system CLK.

sts TCC2_CTRLA, r16           ;

ldi r16, low(TCPER)           ; Load the period of the TC into the TC's period regs and load the same

sts TCC2_LPER, r16            ; value onto the TC's compare A reg.

sts TCC2_LCMPA, r16           ;

ldi r16, high(TCPER)          ;

sts TCC2_HPER, r16            ;

sts TCC2_HCMPA, r16           ;

pop r16                       ; Restore r16.

ret                           ; Return.

H) Appendix:

Files:

- lab2.pdf

- lab2a.asm

- lab2b.asm

- lab2c.asm

- lab2d.asm

Screenshots:

Parts A:

8 MHz clock signal:

32 MHz clock signal:

Part B:

Logic Analyzer displaying the low byte of the TC’s count.